| Subject | FUNCTION KEYS ON MDI KEYBOARD NOT WORKING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product | CNC |  |  |  |  |  |  | Content/Solution |  |  |  |  | Software |  |  |  |  |  |  |
| Controller Model |  |  |  |  | FS-0i-LF PLUS |  |  |  | Controller Sl. No. |  |  |  |  |  | E211A2910 |  |  |  |  |
| Laser/RO/RM Model |  |  |  |  | NA |  |  |  | Laser/RO/RM Sl. No. |  |  |  |  |  | NA |  |  |  |  |
| Machine Model |  |  |  |  | Fl-4020 |  |  |  | Machine Sl. No. |  |  |  |  |  | SIL4020FL0034 |  |  |  |  |
| Machine Tool Builder |  |  |  |  | Suresh Indu Lasers Pvt. Ltd. |  |  |  |  |  | Configuration |  |  |  |  | 4 axis servo |  |  |  |
| End-User |  |  |  |  | GURUKRUPA ENGINEERING WORKS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application |  |  | FIBER LASER |  |  |  |  |  |  |  |  | Installation Date |  |  |  |  |  | August 2021 |  |
| MR No. |  | M66867500& M66867501 |  |  |  | MR Review | Multiple Visit |  |  | Visits Count |  |  |  | 3 |  |  | Hours Count |  | 10 |
| Reason for Multiple/Repeat visits |  |  |  | Problem is intermittent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for presenting this subject |  |  |  | To share information with other zones |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Problem as reported by Enduser |  |  |  | MDI keyboard not working |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observation of the problem with detailed description |  |  |  | Function keys(POS , PROG , OFS , MSG, GRAPH, CUSTOM , SYSTEM and HELP) not working in MDI keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Problem suspected by FSE/SRC |  |  |  | Problem due to hardware |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| History of the problem |  |  |  | NIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Was there any specific external disturbance? |  |  |  | NIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| How many times has it occurred? |  |  |  | Problem is intermittent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Diagnosis information as per manual |  |  |  | NIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| How was the problem analyzed? 1st visit dtd.20-08-2021: - Visited customer & problem not observed during visit, Incoming supply was 397,398 & 398VAC & Earth to neutral 0.4VAC. Control supply 24.2 & 24.3VDC. - As problem was intermittent and taken hardware details. Found earthing to system and MDI keyboard were not connected. Asked customer to connect earth with help from MTB. - As per customer feedback, function keys POS,PROG,OFS,MSG,GRP,CUS & SYS buttons were not working. 2nd Visit dtd. 27-08-2021: - Problem observed during visit. Found Function Keys (POS , PROG , OFS , MSG, GRAPH, CUSTOM , SYSTEM and HELP) were not working in MDI and custom screen as well. - Also when any of the function key is pressed it operates like a cursor key instead of its specific function. - Also, earthing was not provided to controller and PANEL I. Requested MTB to connect the same and checked. found same issue. - Replaced the MDI keyboard with new and observed same problem. - Restored to original keyboard and Performed calibration of touch panel and found same issue. - Discussed with service support team and suspected problem may be due to keyboard file software corruption. - So collected the below software details to update and check the iHMI software iHMI SOFTWARE VERSION: 6 HARDWARE CONFIGURATION 3rd Visit dtd. 27-10-2021: - During visit, observed ALL Function keys and other function keys were not operating. - Also found ALL Function key works like cursor movement and MSG key works as Right click on mouse. - As per guidance from service support disabled mouse keys on control panel. - Path is Control Panel>> Ease of access>>Ease access center>>Make the mouse easier to use>> Control the mouse with the keyboard>>Turn on mouse keys. - Refer below image. - Checked keyboard function and machine power reset done several time no issue.All keys are working normally. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| How was the Problem Solved? - Problem solved after disabling mouse keys option in control panel mouse with the keyboard option. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Justification for Parts replaced: Specification; Qty; Reason. NIL; NIL; NIL. |

Any additional information: Click or tap here to enter text.

For Reviewer’s:

| Applicable for CNC/Servo/Robot/RM Models |  |  |  | CNC,RM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Content useful for Telephonic Support |  | Yes |  |  | Supports MTTR | Yes |
| Any Special Tool used for Troubleshooting |  |  | Click or tap here to enter text. |  |  |  |
| Addition information | Click or tap here to enter text. |  |  |  |  |  |
| Any alternate process to troubleshoot this subject | Click or tap here to enter text. |  |  |  |  |  |
| Remarks | Click or tap here to enter text. |  |  |  |  |  |

| Prepared by | Review by | Presented by | ST Review |
| --- | --- | --- | --- |
| Aravinthkailash & Mahaveer | Kiran V. Raskar | Aravinthkailash & Mahaveer | Click or tap here to enter text. |

+++++End of Report+++++